get/set VGAreg, a simple VGA register hacking program

=====================================================

This little tool is really included here for those who cannot resist to hack

around with their VGA cards, without really having to know how to access

them from Linux.

It should be considered even MORE dangerous than SVGATextMode ;-)

It is FAR from complete, and should get a complete re-write. But it helped

me to test some stuff I needed for SVGATextMode. Anyone tempted to do better

is hereby invited to ? _do_ better.

The syntax is quite easy:

./setVGAreg: version 0.4.3.

Usage: ./setVGAreg [options] VGA_register_set [register_index] data

Options: -n  Don't program VGA hardware

-d  print debugging information

-h  print usage information

-u  unlock chipset-specific registers (needs /etc/TextConfig file)

-p  produce 'pipeable' output (i.e. just the result, no text)

VGA_register_set: any of CRTC, SEQ, ATRCTL, GRCTL, MISC.

register_index: an index in the specified VGA_register_set,

either in decimal (e.g. '24') or hex ('0x18') notation.

Only needed when it is an indexed register.

data: the data to program into the specified register.

./getVGAreg: version 0.4.3.

Usage: ./getVGAreg [options] VGA_register_set [register_index]

Options: -n  Don't program VGA hardware

-d  print debugging information

-h  print usage information

-u  unlock chipset-specific registers (needs /etc/TextConfig file)

-p  produce 'pipeable' output (i.e. just the result, no text)

VGA_register_set: any of CRTC, SEQ, ATRCTL, GRCTL, MISC.

register_index: an index in the specified VGA_register_set,

either in decimal (e.g. '24') or hex ('0x18') notation.

Only needed when it is an indexed register.

An example get/setVGAreg session could be:  ("> " denotes user input, the

rest is output from the program)

> getVGAreg CRTC 0x09

VGA 'CRTC' register, index 9 (=0x9) contains 107 (=0x6b)

> setVGAreg CRTC 0x09 0xeb

VGA 'CRTC' register, index 9 (=0x9) contains 235 (=0xeb)

Which will do something nice to your screen... ;-) (no harm done, don't be

scared).

The '-p' option outputs just one number (the contents of the register). This

is usefull when used in a pipe, or in a script:

#!/bin/bash

ORIG31=`/sbin/getVGAreg -up CRTC 0x31`

echo Original CRTC reg 0x31 contents: $ORIG31

/sbin/setVGAreg -u CRTC 0x31 0x00

/usr/bin/X11/XF86_SVGA

/sbin/setVGAreg -u CRTC 0x31 $ORIG31

WARNING: since different SVGA cards use different extra address ranges in any

of the register sets, no checking is done to make sure you don't attempt to change

a non-existing register!

This is just a hacking tool! Use at your own risk. It was NOT intended to be

idiot proof! If you don't understand all this, then don't bother trying to use it.