NOTES FOR HARDWARE PROGRAMMERS

This file provides info that's missing from trident.doc and

vga.doc.  Those files contain descriptions of the registers, but

not quite enough info to succeed in getting the card to run

properly in the SVGA modes.

WARNING:  I've uncovered this info by digging into the hardware

and making guesses, so it's possible that the intended usage of

anything is not quite like how I've used it or say to use it here.

With that in mind, you need to know the following:

1.  The register values for each mode.

Using MSDOS or whatever, put the card into each video mode using

the BIOS and read these registers:

Attribute controller:  0-0x14

Sequencer:  0-0xe (both old and new registers for 0xd and 0xe)

Graphics Controller:  0-0x8

CRTC:  0-0x1f

Misc Output:  one port

The above doc files will tell you which ports these correspond to.

2.   How to unprotect some regs.

To write to CRTC regs 0-7 one has to clear a bit in register 0x11

(see vga.doc)

To write to sequencer register 0xc it must be unprotected by

setting the high bit in the sequencer's new mode register 0xe.

Reset this bit when done.

3.  Other useful register info

Bit #3 in sequencer register 0xf is set when jumper sw3 is in the

fast scan/non-interlaced position.

This should be enough to get the card working correctly.  Other

details like bank select are covered in trident.doc, vga.doc, and

other literature.

If you're adapting the code for your own purposes, be very careful

about how you're writing the SVGA registers.  The order seems to

be important; you can try rearranging the writes in set_regs() to

see what happens, but do it after you get your code working.